DOE – Selecting a statistical design

To avoid many of the pitfalls of experimentation due to not having incorporated statistical consideration which can lead to the statistical analyses of the results being inconclusive or, worse still, misleading, we have to consider several key criteria in the design of an experiment. Amongst the more important statistical design criteria, we have:

- Consideration of experimental objectives

First and foremost, we should always set goals and determine what variables to measure in an experiment. These considerations will influence both the experimental design and the collection of information on uncontrollable factors. We must anticipate conclusions of the experiment to prevent unexpected complications when the experiment is completed and the research report is being written.

- Factor effects

One should then ask: what is the likely effects of the factors selected?  Of course ideally, inclusion of all relevant factors, when experimentally feasible, is necessary to ensure that uncontrolled systematic variation of these factors will not bias the experimental results. Furthermore, a possible interaction effect of factors cannot be ruled out initially.

- Precision and efficiency

Precision measures the variability of individual responses or the vaiability of effects that measure the influence of the experimental factors. When the standard deviation of a series of responses is sufficiently small, we say the responses are reasonably precise.  Blocking, repeat tests, replication and adjustment for covariates can all increase precision in the estimation of factor effects. For example, blocking increases precision and decreases variability by controlling the systematic variation attributable to nonhomogeneous experimental units or test conditions.  Adjustment for covariates increases precision by eliminating the effects of uncontrolled factors from the variability that would otherwise by attributed to random error.

If a factor is believed to affect the response independently of any other factor or if joint effects of the factors are of secondary interest (as in certain pilot studies), one can carry out a screening experiment as an efficient mean to assess the effects of the factors. If the effect of one factor on the response depends on the levels of the other factors, a larger and more elaborate design is needed.

- Randomization

It is important that we randomize the sequence of test runs or the assignment of factor-level combinations to experimental units.  This is to protect against unknown or unmeasured sources of possible bias such as instrument drift. Randomization of the test runs cannot prevent instrument drift but it can help ensure that all levels of a factor have an equal chance of being affected by the drift. Indeed randomization also helps to validate the assumptions (such as assuming normal distribution) needed to apply certain statistical techniques.

In summary, the following table sets out the useful statistical design criteria:

- Consideration of objectives

- Nature of anticipated conclusions

- Definition of concepts

- Determination of observable variables

- Factor effects

- Elimination of systematic error

- Measurement of covariates

- Identification of relationships

- Exploration of entire experimental region

- Precision

- Estimation of variability (uncertainty)

- Blocking

- Repeat tests, replication

- Adjustment for covariates

- Efficiency

- Multiple factors

- Screening experiments

- Fractional factorials

- Randomization